# MATLAB INTERFACE FOR RPL DODAG VISUALIZATION ( .m FILE)

Steps to run MATLAB interface

- Create MATALB_Interface.c file inside RPL folder, path to this folder <NetSim installed directory>\src\Simulation\

(Note: MATLAB_Interface.c file is provided in the folder)

- Open Nesim.sln, Path toNetSim.sln is <NetSim installed directory>\src\Simulation\

- Right click on RPLAdd Existing Item Select MATLAB_Interface.c file.

- Place PlotDAG.m file inside the installation folder of MATLAB i.e. “C:\Program Files\MATLAB\R2016a”, (Note: PlotDAG.m is provided in the folder)

- Open RPL.c file and add fn_netsim_matlab_init() and fn_netsim_matlab_DODDAG_Init() inside fn_NetSim_RPL_Init() and  fn_netsim_matlab_Finish() inside fn_NetSim_WLAN_Finish ().

- Add definitions of the following functions inside RPL.h file

double fn_netsim_matlab_init();

double fn_netsim_matlab_DODDAG_Init();

double fn_netsim_matlab_DODDAG_run();

double fn_netsim_matlab_finish();

- Go to the Neighbor.c file. Inside Function void choose_parents_and_siblings(NETSIM_ID d) add fn_netsim_matlab_DODDAG_run() below rpl_add_route_to_parent()

10. To compile a MATLAB engine application in the Microsoft Visual Studio 2015 environment, Right click on the RPL project and select PROPERTIES in the solution explorer. Once this window has opened, make the following changes:
     a. Under C/C++ General, add the following directory to the field ADDITIONAL INCLUDE DIRECTORIES:  <Path where MATLAB is installed>\extern\include

b. Under C/C++ Precompiled Headers, select "Not Using Precompiled Headers".

c. Under Linker General, add the directory to the field ADDITIONAL LIBRARY DIRECTORIES:

<Path where MATLAB is installed>\extern\lib\win64\microsoft

d. Under Configuration Properties ->Debugging

Add the following Target path in the Environment:

<Path where MATLAB is installed>\bin\win64

e. Under Linker Input, add the following names to the field marked ADDITIONAL DEPENDENCIES:

libmx.lib; libmat.lib; libeng.lib by separating them with a semicolon.

Click on Apply and then on ok.

f. Make sure that the following directory is in the PATH(Environment variable)

<Path where MATLAB is installed>\bin\win64

(Note: To run this code 64- bit version of MATLAB must be installed in your system. If you are interfacing for the first time then open command window and go to the <NetSim installed directory>\bin and type matlab -regserver)

- Now Right Click on RPL project and select Rebuild.

- Now replace the newly built libRPL.dll from the DLL folder, into the NetSim bin folder. Please ensure you rename the original libRPL.dll file to retain a copy of the original file.

- Run NetSim in Administrative mode. Create a Network scenario IoT.

Multi point to Multipoint Link Properties are set accordingly

Set Velocity to the sensors

And set Receiver Sensitivity as -45

A plot will open, showing the DODAG when the simulation is started and the first route is formed between sink node and the sensor. And the DODAG will be dynamically updated. 
                                              Initially formed DODAG

DODAG formed after some time due to movement in sensors

DODAG at the end of the simulation

After simulation press any key in the NetSim command window to close the MATLAB